- Chapter 16, part 1

- ENGINE ASSEMBLY

Directions:

A.	Use a pencil and Scantron answer form

B.	Write your name on the answer form

C.	Mark the letter choice of the best answer

D.	Do not write on the test

1.	Machinist A says install lip type seals with sealer on the seal lip.  Machinist B says install them with oil around the outside of the case.  Who is right?

a.	Machinist A			c.	Both A and B

b.	Machinist B			d.	Neither A or B

2.	Machinist A says fit rope type rear main seals with a tool matching the journal diameter.  Machinist B says fit them with round stock not larger than the shaft’s seal diameter.  Who is right?

a.	Machinist A			c.	Both A and B

b.	Machinist B			d.	Neither A or B

3.	Machinist A says that after line boring or honing, the seal location in the rear main cap and crankcase is no longer round.  Machinist B says seals in the front and the rear may require centering after line boring or honing.  Who is right?

a.	Machinist A			c.	Both A and B

b.	Machinist B			d.	Neither A or B

4.	Install press-fit oil plugs with

a.	no sealer

b.	sealer around the outer edges

c.	sealer on the backside

d.	sealer on all sides

5.	Machinist A says cotton rope rear main seals come coated with a dry lubricant.  Machinist B says soak these seals in oil prior to assembly.  Who is right?

a.	Machinist A			c.	Both A and B

b.	Machinist B			d.	Neither A or B

6.	Machinist A says to assure longevity of seals on rotating shafts, keep bearing clearance within specifications.  Machinist B says concentricity between the shaft and seal is important to seal longevity.  Who is right?

a.	Machinist A			c.	Both A and B

b.	Machinist B			d.	Neither A or B

7.	To prevent damage to piston skirts in oversized cylinders,

a.	hone to proper surface finish

b.	chamfer the top edges

c.	remove sharp edges on the bottom of the cylinders

d.	chamfer the skirts

8.	Machinist A says correct camshaft binding in new bearings by grinding or polishing cam journals to fit.  Machinist B says allow bearings to run-in in order to correct binding.  Who is right?

a.	Machinist A			c.	Both A and B

b.	Machinist B			d.	Neither A or B

9.	Grease camshaft lobes on assembly to

a.	lubricate the camshaft until the oil pump primes

b.	prevent binding of camshaft in new bearings

c.	prevent scuffing of cam bearings

d.	prevent scuffing of cam lobes during break-in

10.	Correct high camshaft end play by replacing the

a.	camshaft sprocket

b.	camshaft sprocket bolts

c.	thrust plate

d.	camshaft

11.	Check the following during crankshaft installation

a.	rod side clearance

b.	sealing of the rear main

c.	end play and main bearing oil clearance

d.	rod bearing oil clearance

12.	Align main bearing thrust flanges by

a.	tapping the crankshaft forward and backward before

torquing the thrust main cap

b.	machining the thrust side of the cap

c.	assembling bearings over dowels

d.	machining thrust bearing flanges

13.	Machinist A says that Plastigage is the only means of checking bearing clearances. Machinist B says that you can compare bearing ID's to journals or crankpins.  Who is right?

a.	Machinist A			c.	Both A and B

b.	Machinist B			d.	Neither A or B

14.	Before installing a crankshaft, check

a.	straightness, size, oil hole chamfers

b.	oil clearance

c.	end play

d.	rod side clearance

15.	As a precaution against loosening rod nuts,

a.	stake them

b.	torque to yield

c.	Loc-Tite them

d.	assemble with lock washers

16.	Unless noted otherwise, torque fasteners as follows:

a.	dry

b.	wet

c.	lubricate and torque to specifications

d.	lubricate with anti-seize lube and torque to

specifications

17.	Machinist A says correcting excessive rod side clearance can require replacing connecting rods.  Machinist B says this sometimes requires replacing the crankshaft.  Who is right?

a.	Machinist A			c.	Both A and B

b.	Machinist B			d.	Neither A or B

18.	A core hole in an engine block measures 1 5/8 in.  A cup type core plug diameter for this hole will be _______ in.

a.	1 11/16

b.	over 1 5/8

c.	exactly 1 5/8

d.	under 1 5/8

19.	Confirm correct valve timing by checking

a.	timing marks on the damper

b.	part numbers

c.	valve to piston clearance

d.	overlap at TDC exhaust

20.	The crankshaft has too little endplay on assembly.  Machinist A says replace the thrust main bearing.  Machinist B says first loosen the thrust main cap and recheck endplay.   Who is right?

a.	A only				c.	Both A and B

b.	B only				d.	Neither A or B

21.	Do not apply gasket sealer to

a.	intake side of the cylinder head

b.	cork gaskets

c.	coated gaskets

d.	manifold surfaces

22.	Machinist A says seal fasteners extending into oil passages.  Machinist B says seal fasteners extending into coolant.  Who is right?

a.	Machinist A			c.	Both A and B

b.	Machinist B			d.	Neither A or B

23.	Machinist A says measure pistons with tapered skirts between the pin-bore and tip of the skirt.  Machinist B says measure pistons with barrel shaped skirts at the pin centerline.  Who is right?

a.	Machinist A			c.	Both A and B

b.	Machinist B			d.	Neither A or B

24.	Machinist A says forged pistons always require greater clearance than cast pistons. Machinist B says some forged pistons require extra clearance and some do not. Who is right?

a.	Machinist A			c.	Both A and B

b.	Machinist B			d.	Neither A or B

25.	Ring end-gap for a cylinder .001” oversize changes as follows:

a.	.001 in. tighter

b.	.003 in. greater

c.	.001 in. greater

d.	.003 in. per inch of cylinder diameter

26.	Machinist A says that except for hypereutectic pistons, minimum ring gap is .003 in. per inch. of diameter.  Machinist B says twice this amount is typical for hypereutectic pistons.  Who is right?

a.	Machinist A			c.	Both A and B

b.	Machinist B			d.	Neither A or B

27.	The taper on the outside of new compression rings provides all of the following except…

a.	scraping action on the exhaust stroke

b.	scraping action on the intake stroke

c.	an oil film on the compression stroke

d.	aiding in ring seating

28.	Use a ring compressor to

a.	fit rings on pistons

b.	increase ring end gap

c.	guide pistons into cylinders

d.	reduce ring end gaps

29.	Excessive oil pressure can cause

a.	filter or filter seal failures

b.	overheated oil

c.	scored main bearings

d.	out of round cam bearings

30.	Machinist A says grease oil pumps on assembly to speed priming.  Machinist B says that before start up, crank the engine with spark plugs removed and ignition or fuel injection disabled until oil pressure builds.  Who is right?

a.	Machinist A			c.	Both A and B

b.	Machinist B			d.	Neither A or B

31.	Prior to installing heads and tightening head bolts, perform all but which one of the following steps

a.	chamfer bolts holes

b.	lubricate or apply sealer to bolt threads

- lubricate bolt holes

- clean bolt holes

32.	Machinist A says head bolt torque patterns begin at the ends and work towards the center.  Machinist B says this is the pattern for disassembly.  Who is right?

a.	Machinist A			c.	Both A and B

b.	Machinist B			d.	Neither A or B

33.	Machinist A says that to obtain uniform clamping force, tighten head bolts in sequence with a torque wrench.  Machinist B says tightening by degrees of rotation is most precise.  Who is right?

a.	Machinist A			c.	Both A and B

b.	Machinist B			d.	Neither A or B

34.	Machinist A says adjust valve spring compressor stroke to clear valve seals.  Machinist B says adjust compressors to fit spring retainers.  Who is right?

a.	Machinist A			c.	Both A and B

b.	Machinist B			d.	Neither A or B

35.	Machinist A says check rocker arms for right and left offsets on assembly.  Machinist B says check oil holes in rocker shafts for correct direction on assembly.  Who is right?

a.	Machinist A			c.	Both A and B

b.	Machinist B			d.	Neither A or B

36.	Confirm correct valve timing by checking

a.	timing marks on the damper

b.	part numbers

c.	valve to piston clearance

d.	overlap at TDC exhaust

37.	Torque to yield head bolts are

a.	replaced with non-torque to yield bolts

b.	reusable

c.	not reusable

d.	sometimes reusable, sometimes not

38.	Machinist A says pre-oil engines with distributor driven pumps by running the pumps with a drill motor or speed handle.  Machinist B says pre-oil engines with crank driven pumps by removing spark plugs, disabling ignition, and cranking the engine until there is pressure.  Who is right?

a.	A only				c.	Both A and B

b.	B only				d.	Neither A or B

39.	Machinist A says install compression rings with the dot facing up.  Machinist B says install piston rings with the arrow facing forward.  Who is right?

a.	A only				c.	Both A and B

b.	B only				d.	Neither A or B

40.	Machinist A says that when installing an exchange crankshaft, check that the flywheel or flexplate and torque converter fit.  Machinist B says that the exchange crankshaft may require rebalancing.  Who is right?

a.	A only				c.	Both A and B

b.	B only				d.	Neither A or B

41. 	When testing a short block in an engine run-in stand, all of the following can be  checked except. . .

- lifter rotation			c. compression

- piston noises			d. the ability of rings to scrape oil