Chapter 2

MACHINING FUNDAMENTALS

Directions:

- Use a pencil and Scantron answer form

- Write your name on the answer form

- Mark the letter choice of the best answer

- Do not write on the test

- Machinist A says that drills and reamers cut from the end only.  Machinist B says that these tools follow the alignment and position of predrilled holes.  Who is right?

a.	A only				c.	Both A and B

b.	B only				d.	Neither A or B

2.	Machinist A says that cutting tools cannot be harder than the material machined. Machinist B says that there must be relative motion between the work and cutting tools for chips to form.  Who is right?

a.	A only				c.	Both A and B

b.	B only				d.	Neither A or B

3.	When turning or boring, a .025 in. depth of cut changes the diameter ______ in.

a.	.0125

b.	.025

c.	.0375

d.	.050

4.	Machinist A says that feed rates for milling cutters are in inches per revolution.  Machinist B says that feed rates for boring or turning are in inches per minute.  Who is right?

a.	A only				c.	Both A and b

b.	B only				d.	Neither A or B

5.	As diameter increases, ______ RPM to maintain correct cutting speeds.

a.	maintain

b.	reduce

c.	increase

d.	adjust according to materials

6.	Machinist A says that rake angles direct chips off of the work.  Machinist B says that rake angles are smallest for boring tools.  Who is right?

a.	A only				c.	Both A and b

b.	B only				d.	Neither A or B

7.	Machinist A says that a smaller nose radius on a single pointed tool improves surface finishes.  Machinist B says that reduced feed rates improves surface finishes.  Who is right?

a.	A only				c.	Both A and b

b.	B only				d.	Neither A or B

8.	Machinist A says that all end-milling cutters cut from the sides.  Machinist B says that only two-lip end mills bore from the end.  Who is right?

a.	A only				c.	Both A and b

b.	B only				d.	Neither A or B

9.	The spindle axis of a surfacing machine is

a.	square with the machine

b.	square with the work

c.	square with the work travel

d.	tilted in the direction of travel

10.	Machinist A says that center drills hold location better because of their short length.  Machinist B says that center drills form a 60-degree included angle for setting up shafts between centers.  Who is right?

a.	A only				c.	Both A and b

b.	B only				d.	Neither A or B

11.      Drilling operations such as reaming, counter-boring, and counter-sinking are done at ______ drilling speed.

a.	half

b.	full

c.	double

d.	triple

12.	Hole alignments or positions are changed using

a.	drills

b.	reamers

c.	hones

d.	boring tools or end mills

13.	Machinist A says that grinding coolants flush away grit and cool both the wheel and the work.  Machinist B says that cutting oils lubricate the cutting action and prevent chip welding on cutting edges.  Who is right?

a.	A only				c.	Both A and b

b.	B only				d.	Neither A or B

14.	Machinist A says that evidence of soft grinding action is a burned surface on the work.  Machinist B says that evidence of hard grinding action is a scored finish.  Who is right?

a.	A only				c.	Both A and B

b.	B only				d.	Neither A or B

15.	Machinist A says that to reduce tool chatter, decrease speeds.  Machinist B says that to reduce chatter, increase feeds.  Who is right?

a.	A only				c.	Both A and b

b.	B only				d.	Neither A or B

16.	For drilling a 3/8-in. hole in steel, what RPM is correct?

a.	1060

b.	 530

c.	 150

d.	 100

17.    Run a 1/2-in. reamer at ______ RPM  in cast iron.

- 480

- 240

- 120

- 60