64HP Study Guide

- 4 stroke theory

Valve events in relation to piston position

Overlap, blowdown

- Formulas

Static compression ratio = Volume at BDC compared to volume at TDC

Effective compression ratio = Volume at IVC compared to TDC

VE:  The ratio between the amount of air an engine consumes while it is running compared to the amount that would fill its displacement completely to atmospheric pressure at that RPM.

  - A measure of an engine’s ability to breathe

  - Peak VE usually occurs at peak torque.  High HP engines, VE occurs at higher RPM

  - Passenger car engines are about 80%

- Blueprinting and machining processes

Bloc-Tru for 90 degree blocks

- Air Flow

  - Types of measurement:

Sharp edge orifice – Superflow.  Flow is measured through an orifice in CFM.

Pitot tube for measuring velocity – compares static and dynamic pressures (V=0 at walls)

  - Set-up

Testing is done steady state, but really flow is pulsed

3 lb. springs (area x diff pressure)

Spark plug installed

Opening fixture and dial indicator

Entry conditions (radius, manifold when possible)

Open valves at consistent intervals

Test pressure, select channel for valve opening

FlowCom

  - Facts

65%-75% exh-to-int flow on 2 valve heads, 75%-85% on 4 valve heads

Some surface roughness can improve flow by delaying separation (golf balls, neg dimples)

Some heads have obstructions by design (GM 60 V-6)

Laminar flow vs. separation (turbulence)

- Combustion, normal and abnormal, mixture motion (stabilize burn rate)

  - Emissions originate in the flame quench area

  - CO

  - HC

  - NOX –

  - CO2 –

  - O2 –

  - Pre-ignition

    - Hot spots (exhaust valve, carbon, sharp edges) causing ignition before spark

    - Max cylinder pressure before TDC

    - Burned area in center of the piston as flames meet

  - Detonation

    - Unburned gasses, high pressure and temp, cause post ignition (after TDC)

    - This second flame front meets the original and causes resonance waves (3000 ft per sec)

    - Burned areas on side of piston, usually on exhaust side

  - Chamber designs

    - Pentroof

4 valve head

Plug centrally located – Less distance to travel, equal to all areas of chamber

Large quench area around

- Hemispherical

Large valve diameters

Large dome on piston is required – Surface area absorbs heat instead of using it

- Wedge – Open vs. Closed

Closed – Small chamber around the valves and plug

Has quench area

Open – Chamber is opened up to the size of the bore

Quench area becomes the part of the roof that is closest to piston

  - Quench

Promotes turbulence for complete combustion

A blast of air forces the flame front through the chamber

  - 2 vs. 4 valve heads

    - 2 small valves flow better because of the higher velocity in smaller ports

    - 4 valve heads have more valve area (curtain)

    - Airflow begins and peaks earlier in 4 valve head

    - Exhaust flow is better – Less lift, duration, and overlap

  - Mixture motion – To stabilize burn rates.  Created at the expense of air flow

    - Faster burn rate

    - More complete combustion at idle and low RPM

    - More consistent burn @ WOT

    - Decreases VE

    - Swirl – 2 valve heads

    - Tumble – 4 valve heads

- Pistons

  - Terms – Crown, dome, valve relief, compression height, pin offset, ring lands & grooves, accumulator groove, skirt, gas porting, anti-detonation grooves

  - Cast pistons – Expansion slots to isolate skirt from crown

  - Forged pistons – No slots so heat goes down through skirt = more clearance needed

  - Hypereutectic 16% silicon – strong, thermally stable, reflective

  - Pin offset for quieter operation, thrust side (not racing pistons though)

  - Pressed fit and full floating

- Rings – Sealing (1000psi), cooling, lubrication

  - Seal combustion up and oil down

  - Gas pressure forces ring out and down

  - Plateau hone to reduce break-in

  - Torque plate and bolt-on items

  - Ring flutter – When blow by starts to unseat the top ring

  - Material

    - Ductile on top in case detonation (nitrided strong but expensive)

    - Gray iron on second groove

    - Stainless expander with chrome rails

    - Molybdenum for wear, chrome for embedibility

  - Side clearance .001-.002, zero back clearance

  - End gap .003-.004 per inch of bore is standard.  Light check

  - Finishes for rings:  Cast & Chrome = 25RA, Moly = 15RA

- Fuels – This outline plus Rick Gold from ERC

  - Fuel is used for heat energy to make mechanical power

  - Volatility – The ability to vaporize

    - Reid Vapor Pressure – Measured at 100F at 1 atmosphere in PSI

    - Distillation profile – Percent of volume a gas evaporating at increasing temperature

      - Front end volatility

        - Cold and hot starting

        - Prevent vapor lock

        - Low evaporation and exhaust emissions

      - Midrange volatility

        - Quick warm up and smooth running

        - Good short trip fuel economy

        - Good power and acceleration

        - Prevents carb icing or hot stalling

      - Tail end volatility

        - Good fuel economy after warm up

        - Prevents engine deposits

        - Minimal dilution of crankcase oil

        - Minimal VOC emissions

- Connecting rods

  - Materials

    - 4340 steel - most economical for high HP applications, high tensile strength,

Nickel-Chromium-Molybdenum (43xx) with 0.40% (xx40) Carbon by weight.  Heated to 2200F and hammered to shape,

    - Powdered Metal rods – Most new OE, closer tolerance on size and weight variation, reduced manufacturing costs, even grain structure,

    - Titanium - expensive ($600 per rod) light weight and strong, about 33% lighter than steel, not needed in street engines, sensitive to scratches and is gumming when rubbed,

    - Aluminum – Disposable, has a dampening effect for high boost or nitrous, requires more mass to maintain the strength (needs space)

  - H beam – Aftermarket, stronger design, expensive to manufacture

  - I beam – OEM connecting rods, slightly lighter, can handle up to 500-700HP,

- Bearings

  - Eccentric to account for rod stretch.  Also provides an oil reservoir

  - Crush height about .002”-.003” forces bearing against housing preventing spinning

  - Outward pressure also ensures heat transfer

  - Conformability, embedability, wear, strength, corrosion resistance

  - Tri-metal bearings have steel backing, copper-lead layer, then lead-tin-copper alloy (Babbitt) layer

    - These overlays are thinner for better support, but don’t last as long

  - Dry film coatings reduce friction and reduce wear due to dry-starts

- Forced induction

  - Turbochargers

  - Wastegate to control boost

  - Terms - Turbine, compressor, blow-through, draw-through, back-pressure,

  - Superchargers – Different types, positive displacement, centrifugal, crank-driven

- Balancing

  - Rotating weight: Big end of rod, bearings

  - Reciprocating weight: Small end of rod, piston, pin, rings, locks

  - Equalize rotating and reciprocating weights

- Intake size

  - Port diameter should be between 80% and 85% of the seat diameter.  The area just under the seat should be about 88% to 93% of the seat diameter.

  - In two valve heads, the intake valve diameter should be about half the bore

  - To figure intake lift, the curtain area should equal valve head area (valve diameter/4)

  - Smaller intake ports build velocity quicker

    - Better atomization

    - RPM range that makes power is wider

    - Better cylinder filling

  - Larger intake ports flow more at high RPM

    - Engines have to be able to rev high enough to take advantage of this flow

  - Carburetor or throttle body size should be RPM x cubic inches/3456=CFM (2 cubic feet in cubic inches)

  - Dual plane manifolds = long runners for increased velocity, less cylinder robbing

  - Single plane manifolds = short runners for high RPM, tougher to get slugs of air moving at low RPM

  - Import manifolds need a radius at runner entry and plenum that extends beyond last cylinder

  - 1 to 1.2 cylinder volume works well for a wide torque curve (volume from back of valve-to-plenum)

  - Expansion waves propagate back in the intake.  When reflected back from the plenum, these positive pressure waves can aid the end of the intake stroke.

- Exhaust size

  - Exhaust port should flow about 70% of the intake

  - The exhaust valve diameter should be about 70% of the intake diameter (Makes the area about half the area of the intake)

  - Too large of an exhaust port slows velocity, decreases the intake initiation (if duration is high, can over-scavenge)

- Exhaust pipes

  - Diameter = exhaust valve diameter + .125”

  - Primary tube length = 2 cylinder volumes long – port volume

  - 4-into-1 collector with a pyramid between pipes to fill dead space

  - When pressure waves heading back to each cylinder aid in reducing exhaust port pressure near the end of the exhaust stroke, it is considered to be tuned.

- Nitrous Oxide (N2O)

  - Air = 21% Oxygen, N2O = 36% Oxygen

  - More air, more fuel needed (high octane)

  - Also provides a cooler charge, which makes it denser

    - When the liquid N2O is vaporized it absorbs the latent heat of vaporization

    - Heating the nitrous closer to intake air temp helps cool charge

  - At 570F, Oxygen and Nitrogen dissociate so oxygen is ready for combustion

  - Nitrous speeds up combustion, timing should be retarded

  - The nitrogen acts as a buffer to combustion, pure oxygen would be too harsh of a flame front

  - N2O with hydrogen peroxide makes the burn harder